NetWare Driver Installation 		May 1992

===========================

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory	      Contents

---------	      --------------------------------------------------------

\NETWARE	      A pre-generated IPX.COM

\NETWARE\V215b	      NetWare v2.15b workstation drivers

\NETWARE\V215C	      NetWare v2.15c, v2.2 or NetWare 386 workstation drivers

\NETWARE\ODI.DOS      ODI workstation driver for DOS

*********************************

*  QUICK SETUP FOR WORKSTATION	*

*********************************

A pre-generated workstation driver IPXE35.COM is included in this disk.

You can simply run the IPXE35.COM on a workstation to access the Novell network:

>IPXE35	  --Supports NetWare Lite, 2.1x, 2.2 or NetWare 386

>NETx	  --This program is provided by Novell

***********************

*  WORKSTATION SETUP  *

***********************

If you want to generate your own workstation driver instead of using the

provided IPXE35.COM, please do the following:

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_DLK.  You can use the DOS "Label" command as follows:

>Label  LAN_DRV_DLK

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy  A:\NETWARE\V215c\*.*  A:\	--For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides in the SHGEN-1 or

WSGEN-1 diskette.  When you are asked to select a network driver, press the

INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert this

diskette (or backup) in drive A.  Upon completion, a new IPX.COM file will be

created in your SHGEN-1 or WSGEN-1 diskette.

************************

*  NETWARE LITE USERS  *

************************

Suppose you have completed the NetWare Lite installation. Use a editor to change

the device driver name that saved in batch file STARTNET.BAT of NetWare Lite

package.

:				:

LSL.COM		change to    LSL.COM

(device driver)   ---------->   E3500ODI.COM

IPXODI.COM 		     IPXODI.COM

:				:

:				:

Then you can type startnet.bat at DOS prompt to startup NetWare Lite.

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

ODI Driver program allows you to run multiple protocl stacks.  It also

allows you to install up to two network adapters in a workstation. ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.	For detailed info on Keywords,

please refer to Novell manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the E3500ODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the E3500ODI.COM whenever a reference is made to the Hardware Specific Module.

3)  A typical startup for ODI workstation is as follows:

>LSL		--Link Support Layer Module provided by Novell

>E3500ODI		--Hardware Specific Module

>IPXODI		--IPX Protocol Stack Module provided by Novell

>NETx		--DOS Support Module provided by Novell

>LOGIN