NDIS Driver Program Installation		May 1992

================================

ETHER-3500 NDIS driver program for DOS follows the Microsoft's NDIS spec v2.0.1.

The NDIS driver program support the following network operating systems:

MS LAN Manager v2.0

DECnet PathWork v4.0

WIN/TCP PathWay Access for DOS v1.1

3COM 3+ Open v1.1

Banyan VINES v4.0

IBM LAN Support Program v1.2.1

***********************************************

*  How to install E3500.DOS on MS LAN MANAGER *

***********************************************

1)  Prepare a diskette with volume name "DOS_DRV_DLK" and create directories

in the following manner:

>label a: DOS_DRV_DLK	  --insert a blank disk in drive 'A'

>md MSLANMAN.DOS	  --make a directory called MSLANMAN.DOS

>cd MSLANMAN.DOS

>md DRIVERS		  --make a directory called DRIVERS

>cd DRIVERS

>md ETHERNET		  --make a directory called ETHERNET

>md NIF 		  --make a directory called NIF

>cd ETHERNET

>md E3500		  --make a directory called E3500

MSLANMAN.DOS  (root)

|

DRIVERS

________|_________

|		  |

ETHERNET		 NIF

|

-------------

|	     |

E3500

2) Insert the ETHER-3500 driver program diskette in drive 'A' and insert the

prepared diskette (DOS_DRV_DLK) in drive 'B'.  Afterwards, copy the

appropriate NDIS driver files from drive 'A' to 'B'.

>a:			--go to drive 'A'

>cd ndis\dos

>copy E3500.DOS     B:\MSLANMAN.DOS\DRIVERS\ETHERNET\E3500

>copy PROTOCOL.INI  B:\MSLANMAN.DOS\DRIVERS\ETHERNET\E3500

>copy E3500.NIF     B:\MSLANMAN.DOS\DRIVERS\NIF

3) MS LAN Manager provides a SETUP.EXE program on their Setup disk.

Execute this SETUP program to specify the proper driver for ETHER-3500 card.

-If you are installing LAN Manager for the first time, do the following:

Upon completion of retrieval of the last workstation's disk contents, you will

be asked if you want to import additional network drivers.  Answer "YES".

-If you have already installed LAN Manager in your hardisk, do the following:

Select the Configuration menu, and then select "Add New Config" option.

Afterwards, choose the "Other Driver" option.

4) Insert the prepared diskette (DOS_DRV_DLK) in drive 'A' and follow the

instructions on the screen.  The SETUP program will copy the necessary

NDIS driver programs from the DOS_DRV_DLK diskette to your hardisk (into the

LAN Manager directory).

5) After completion of the SETUP, your CONFIG.SYS file will be modified and

contain statements referring to the ETHER-3500 NDIS driver programs.

Reboot your computer and follow the NDIS binding procedure to use the

ETHER-3500 NDIS driver programs.

*******************************************************

*  How to install E3500.DOS on WIN/TCP PathWay Access *

*******************************************************

1) Copy E3500.DOS to directory C:\LANMAN.

2) Go to the directory C:\LANMAN, and create a text file called PROTOCOL.INI:

[E3500]

drivername = E3500$

ioaddress = 0x300

interrupt = 3

3) Add the following statements to your CONFIG.SYS file:

device=c:\lanman\E3500.DOS

4) Add the following statements to your AUTOEXEC.BAT file:

ndis -d:1

winbind

wintcp

****************************************************

* How to install E3500.DOS on DECnet PathWork v4.0 *

****************************************************

You can use the DECnet PathWork's  NETSET.EXE program to install NDIS

driver program. Please refer to DECnet installation manual for detailed

information.  The following instructions assume that you have already

installed the DECnet PathWork with 3Com's Etherlink II card.

1) Modify your CONFIG.SYS file as follows:

device = c:\decnet\ELNKII.SYS

to

device = c:\decnet\E3500.DOS

2) Modify the PROTOCOL.INI file as follows:

from:

---------------------------------

[ELNKMACII]

drivername = ELNKII$

ioaddress = 0x300

interrupt = 3

xmitbufs = 2

[IPX4MAC]

:

bindings = ELNKMACII

:

[DATALINK]

:

bindings = ELNKMACII

:

to:

-----------------------------------

[E3500_NIF]

drivername = E3500$

ioaddress = 0x300

interrupt = 3

[IPX4MAC]

:

bindings = E3500_NIF

:

[DATALINK]

:

bindings = E3500_NIF

:

********************************************

* How to install E3500.DOS on 3Com 3+ Open *

*********************************************

Supposing that you have installed the 3Com 3+ Open with 3Com's 3C501 card:

1) Copy E3500.DOS to  C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS.

2) Modify the device driver statement for 3C501 card in your CONFIG.SYS to:

device = c:\3open\doswksta\lanmam\driver\E3500.DOS

3) Modify the PROTOCOL.INI file to:

[ETHERLINK]

:

drivername = E3500$

ioaddress = 0x300

interrupt = 3

:

*************************************************

* How to install E3500.DOS on Banyan VINES v4.0 *

*************************************************

1) Create a bootable diskette using DOS 3.1 or above.  Insert this diskette

in drive 'A'.

2) Login to your VINES network.

3) Run PCCOPY.	 Select 'NDIS Ethernet" from PCCOPY menu. PCCOPY will copy

the NDIS-specific files to the bootable diskette.

4) Copy the E3500.DOS file from ETHER-3500 driver diskette to the bootable disk.

5) Create a CONFIG.SYS file which contains the following statements:

device = protman.dos

device = E3500.DOS

6) Create a PROTOCOL.INI file as follows:

[PROTOCOL MANAGER]

drivername = PROTMAN$

[VINES_XIF]

drivername = NDISBAN$

bindings = "E3500_NIF"

[E3500_NIF]

ioaddress = 0x300

interrupt = 3

drivername = E3500$

7) Run PCCONFIG, and select the Network Card Setting.

8) Choose the 'NDIS Ethernet' from the menu.   Afterwards, enter the correct

hardware settings.

9) Save the new value and exit from PCCONFIG.

10) Reboot your computer.

**************************************************************

* How to install E3500.DOS on IBM LAN Support Program v1.2.1 *

**************************************************************

Supposing that you have installed the IBM LAN Support Program in your computer,

you can use E3500.DOS to replace the installed driver program. Follow the

instructions below to change the contents of CONFIG.SYS and PROTOCOL.INI files.

1. Copy E3500.DOS to the directory where your IBM LAN Support program resides.

2. Modify the original NDIS device driver statement in CONFIG.SYS as follows:

DEVICE = [driver]:\path\E3500.DOS

3. Modify the PROTOCOL.INI file as follows:

BINDINGS = E3500

:

:

[E3500]

DRIVERNAME = E3500$

IOADDRESS = 0x300

INTERRUPT = 3

==============================================================================

ETHER-3500 NDIS PROTOCOL.INI User Guide

The keywords used to create the PROTOCOL.INI file for ETHER-3500 is as follows:

IOADDRESS - This specifies the base I/O address used by ETHER-3500 card.

This value must match the actual base I/O setting of your card.

INTERRUPT - This specifies the interrupt used by the ETHER-3500 card.

This value must match the actual IRQ setting of your card.

DRIVERNAME -This specifies the network card's driver name.

BASEMEMORY -This specifies the base memory address used by the ETHER-3500

cards.

This value must match the base memory setting of your card.

NETADDRESS -This specifies the node address (hex) used by your network card.

However, you can specify a node address different from the one

that is burned into your network card.  In such case, the user

specified node address will be used instead of the card's

actual address.