5.0 CREATING CUSTOM LIBRARY PARTS WITH OrCAD/SDT

Creating "custom" libraries to run with  OrCAD/SDT  is  a  simple  two  step

process.   With a standard  text editor, symbols are  created using a Symbol

Description  Language.  This language is so simple, that you most likely can

learn it  within ONE HOUR.   After the library parts  are represented in the

text  source file, the second step is to compile it using a Library Compiler

that is included with OrCAD/SDT.

You  may  also  modify  OrCAD's  existing  library  parts  using  a  library

de-compiler program.  This automatically generates a working source file for

you to modify.   Since  the  Library  Compiler  is  not  included  with  the

demonstration  disk, we will illustrate the  procedure for creating a couple

of block and bitmap library parts.

5.1  SAMPLE SOURCE FILE USING THE SYMBOL DESCRIPTION LANGUAGE

In this example library source  file,  we  will  illustrate  how  to  create

library  parts with OrCAD/SDT.  The power of this method in creating library

parts  is further illustrated when parts  have multiple devices per package.

'2114' '2148' {1Kx4 SRAM}

6   14  1

L1  5   IN   'A0'

L2  6   IN   'A1'

L3  7   IN   'A2'

L4  4   IN   'A3'

L5  3   IN   'A4'

L6  2   IN   'A5'

L7  1   IN   'A6'

L8  17  IN   'A7'

L9  16  IN   'A8'

L10 15  IN   'A9'

L12 8   IN   'C\S\'

L13 10  IN   'W\E\'

R1  14  HIZ  'D0'

R2  13  HIZ  'D1'

R3  12  HIZ  'D2'

R4  11  HIZ  'D3'

T0  18  PWR  'VCC'

B0  9   PWR  'GND'

DISCUSSION:  All part names are enclosed within  a single quote.  The second

row contains  information on the size  of the part.   The "6" represents the

parts  size along the "X" coordinate axis, and the "14" represents the parts

size  along the "Y" coordinate axis.  The "1" represents the number of parts

that  are in  the package.   Since  there is  only one  memory chip  in this

device,  a "1" is placed at this location.  Pins may be placed on any of the

four sides  of a part.   If you  want pins placed  on the left  side the "L"

token is used.  The right side uses "R", the "B" and "T" are used for bottom

and top.

In  this example: "L1" places a pin on  the left side, at the first position

down from the top.   The "5"  is the actual  pin number from  the data book.

"IN" indicates the  this pin  is a  input.   Other pin  types in  the Symbol

Description Language include:  OUT,  I/O,  OC  (open  collector),  PWR,  PAS

(passive),  SHORT (short leads), and HIZ.  Finally, the Pin Name is enclosed

within  single quotes.  A back slash "\" places a bar over the letter in the

Pin Name. Comments are enclosed within brackets {...}.

This process is repeated for every pin.

'4027'

6    6    2

L1   6    10   IN      'J'

L3   3    13   CLK IN  'CLK'

L5   5    11   IN      'K'

R1   1    15   OUT     'Q'

R5   2    14   OUT     'Q\'

T3   7    9    IN      'PR'

B3   4    12   DOT IN  'R'

T0   16   16   PWR     'VDD'

B0   8    8    PWR     'VSS'

DISCUSSION: The "CLK" token  places the  clock symbol  at the  specified pin

location.   Also notice that this device has two parts per package.  This is

accounted with the "2" at the parts per package location.  The second column

represents the pin numbers of  the  first  package,  and  the  third  column

represents the pin numbers of the second package.  The "DOT" token places an

inversion bubble at the specified pin location.

THE FOLLOWING ARE EXAMPLES OF CREATING BITMAP SYMBOLS:

'CAPACITOR'

2   3   0

T1  SHORT PAS 'A'

B1  SHORT PAS 'B'

{00}..........#

{01}..........#

{02}..........#

{03}.###################

{04}.

{05}.

{06}.

{07}.

{08}.....###########

{09}...###....#....###

{10}.##.......#.......##

DISCUSSION: Bitmap symbols use  a  combination  of  the  Symbol  Description

Language and  bitmap representations.   A  Period "."  placed in  the bitmap

represents a pixel  bit that is not  displayed on the screen.   A Pound Sign

"#" represents a  set pixel bit.   Each pixel bit placed  in the source file

represents  a screen spacing of 1/100th of an inch.  Pins may be placed on a

bitmap every 10 pixels, or 1/10th inch apart.  In the case of the Capacitor,

pins are placed at "T1" and "B1".  When a zero ("0") is placed in the number

of pins per package location, Pin Numbers are not placed on the part when it

is drawn.

'LED'

4   2   0

L1  PAS  'ANODE'

R1  PAS  'CATHODE'

{00}............................#........#

{01}.......#.........#........###......###

{02}.......##........#......#####....#####

{03}.......###.......#....#######..#######

{04}.......####......#......#........#

{05}.......#####.....#.....#........#

{06}.......######....#....#........#

{07}.......#######...#...#........#

{08}.......########..#..#........#

{09}.......#########.#

{10}#########################################

{11}.......#########.#

{12}.......########..#

{13}.......#######...#

{14}.......######....#

{15}.......#####.....#

{16}.......####......#

{17}.......###.......#

{18}.......##........#

{19}.......#.........#